ELEVATE LABS
Cyber Security Internship

Task 5
Documentation Report
Submitted by Suhaila P.S

Network Traffic Analysis with Wireshark

1. Introduction

This report documents the systematic process of capturing, filtering, and analyzing live network traffic using Wireshark on Kali Linux, as per the task requirements. Key objectives included:

- Packet Capture: Live traffic interception on interface eth0.

- Protocol Identification: Isolation of TCP, ICMP, and DNS protocols.

- Export & Analysis: Saving captures as .pcap and summarizing findings.

2. Methodology

Step 1: Setup & Verification

- Tool: Wireshark (Kali Linux pre-installed).

Figure 1: Terminal output showing Wireshark version.

Step 2: Traffic Capture

- Interface: eth0 (Ethernet).

- Traffic Generation:

- ICMP: ping google.com.

- HTTP/TCP: Browsing http://example.com.

- DNS: Automatic queries during browsing.

Figure 2: Wireshark’s live capture on eth0 with traffic spikes.

Step 3: Protocol Isolation

| Filter | Example Packet | Purpose |
| --- | --- | --- |
| tcp | Packet #33 (10.0.2.15 → 34.36.137.293:448) | Web connections. |
| icmp | Ping replies from 142.250.205.14 | Network testing. |
| dns | Queries to 192.168.1.1 | Domain resolution. |

Figure 3: Filtered tcp view

Figure 4: Identified TCP, ICMP, and [HTTP/DNS]

Step 4: Export & Preservation

- Action: Saved as task5_capture.pcap.

Figure 6: File explorer with .pcap file.

3. Findings & Analysis

Key Observations

- TCP Dominance:

  - 60% of traffic comprised TCP handshakes (e.g., SYN, ACK flags).

  - Example: Packet #33 to port 448 (likely a service or typo).

- DNS Workflow:

  - Repeated queries to 192.168.1.1 (local DNS resolver).

  - Packet #32 contained a 172-byte DNS response.

- ICMP Utility:

  - Ping RTT (Round-Trip Time) averaged 20ms to Google.

5. Conclusion & Recommendations

- Achievements:

  - Captured and analyzed 3+ protocols (TCP, ICMP, DNS).

  - Preserved raw data in .pcap for future analysis.

- Improvements:

  - Filter http to inspect unencrypted web traffic.

  - Capture ARP or HTTPS for advanced analysis.

Report Author: Suhaila P.S
Date: [02/06/2025]